## Initial Setup of Jetson Orin Nano

https://developer.nvidia.com/embedded/learn/get-started-jetson-orin-nano-devkit#intro

## SSH

sudo systemctl enable ssh

sudo systemctl start ssh

ip a or ip addr —to check ip address

ssh-keygen -R ip address — to remove prev key

## Camera Setup

sudo  apt update

sudo apt upgrade

sudo /opt/nvidia/jetson-io/jetson-io.py

This opens the Jetson-IO Configuration Tool

You’ll see a menu like this:

Jetson-IO

├── Configure Jetson 40pin Header

├── Configure CSI Connector

├── Save and reboot

Choose:

Configure CSI Connector

Then:

- Select Camera

- Select IMX477 (or Arducam IMX477 if listed)

- Enable it

Finally:

Save and reboot

Reboot is mandatory

To open camera

gst-launch-1.0 nvarguscamerasrc ! nvvidconv ! autovideosink

YOLO V11

Run on JetPack 6.1

Installing Ultralytics Package

We will install Ultralytics package on the Jetson with optional dependencies so that we can export the PyTorch models to other different formats.

Update packages list, install pip and upgrade to latest

sudo apt update

sudo apt install python3-pip -y

pip install -U pip

Install ultralytics pip package with optional dependencies

pip install ultralytics[export]

Reboot the device

sudo reboot

Install PyTorch and Torchvision

Install torch 2.5.0 and torchvision 0.20 according to JP6.1

pip install https://github.com/ultralytics/assets/releases/download/v0.0.0/torch-2.5.0a0+872d972e41.nv24.08-cp310-cp310-linux_aarch64.whl

pip install https://github.com/ultralytics/assets/releases/download/v0.0.0/torchvision-0.20.0a0+afc54f7-cp310-cp310-linux_aarch64.whl

Install cuSPARSELt to fix a dependency issue with torch 2.5.0

wget https://developer.download.nvidia.com/compute/cuda/repos/ubuntu2204/arm64/cuda-keyring_1.1-1_all.deb

sudo dpkg -i cuda-keyring_1.1-1_all.deb

sudo apt-get update

sudo apt-get -y install libcusparselt0 libcusparselt-dev

For JetPack 6 with Python 3.10 support, install onnxruntime-gpu 1.23.0:

pip install https://github.com/ultralytics/assets/releases/download/v0.0.0/onnxruntime_gpu-1.23.0-cp310-cp310-linux_aarch64.whl

Convert Model to TensorRT and Run Inference

The YOLO11n model in PyTorch format is converted to TensorRT to run inference with the exported model.

from ultralytics import YOLO                                                                                                                                                import cv2                                                                                                                                                                                                                                                                                                                                              # 1. Load TensorRT engine                                                                                                                                                   print("[INFO] Loading YOLO11n TensorRT engine...")                                                                                                                          model = YOLO("yolo11n.engine", task="detect")  # make sure this file is in the same folder                                                                                  print("[INFO] Engine loaded.")

gst = (                                                                                                                                                                         "nvarguscamerasrc sensor-id=0 ! "                                                                                                                                           "video/x-raw(memory:NVMM), width=1280, height=720, format=NV12, framerate=30/1 ! "                                                                                          "nvvidconv ! video/x-raw, format=BGRx ! "                                                                                                                                   "videoconvert ! video/x-raw, format=BGR ! "                                                                                                                                 "appsink drop=1"                                                                                                                                                        )                                                                                                                                                                                                                                                                                                                                                       print("[INFO] Opening camera with GStreamer...")                                                                                                                            cap = cv2.VideoCapture(gst, cv2.CAP_GSTREAMER)                                                                                                                                                                                                                                                                                                          if not cap.isOpened():                                                                                                                                                          print("[ERROR] Failed to open IMX477 via GStreamer. Check pipeline / OpenCV build.")                                                                                        raise SystemExit                                                                                                                                                                                                                                                                                                                                    print("[INFO] Camera opened successfully.")                                                                                                                                                                                                                                                                                                             # 3. Live loop                                                                                                                                                              while True:                                                                                                                                                                     ret, frame = cap.read()                                                                                                                                                     if not ret:                                                                                                                                                                     print("[ERROR] Failed to read frame from camera.")                                                                                                                          break                                                                                                                                                                                                                                                                                                                                               # YOLO11n inference, only 'person' class (0)                                                                                                                                results = model(frame, imgsz=640, device=0, classes=[0])                                                                                                                    annotated = results[0].plot()                                                                                                                                                                                                                                                                                                                           cv2.imshow("YOLO11n TensorRT - IMX477", annotated)                                                                                                                                                                                                                                                                                                      if cv2.waitKey(1) & 0xFF == 27:  # ESC to exit                                                                                                                                  break                                                                                                                                                                                                                                                                                                                                           cap.release()                                                                                                                                                               cv2.destroyAllWindows()                                                                                                                                                     print("[INFO] Stopped.")

Export YOLO11n → TensorRT engine (FP16)

Run this once:

yolo export model=yolo11n.pt format=engine device=0 half=True imgsz=640

Run  script

python3 yolo11.py

### Pixhawk & Jetson Connection

Hardware Connection

| Pixhawk TELEM2 | Jetson Orin Nano |
| --- | --- |
| TX (Pin 2) | RX (Pin 10) |
| RX (Pin 3) | TX (Pin 8) |
| GND (Pin 6) | GND (Pin 6) |
| 5V (Pin 1) | ❌ NOT CONNECTED |

Jetson Orin System Setup

Enable UART

sudo systemctl stop nvgetty

sudo systemctl disable nvgetty

sudo usermod -aG dialout $USER

reboot

Install Required Libraries

sudo apt-get update && sudo apt-get install -y \

python3-dev \

python3-opencv \

python3-wxgtk4.0 \

python3-matplotlib \

python3-lxml \

libxml2-dev \

libxslt-dev

MAVLink libraries

pip3 install pymavlink mavproxy

Heartbeat test

mavproxy.py --master=/dev/ttyTHS1 --baudrate 115200

Expected Output

Waiting for heartbeat from /dev/ttyTHS1

Detected vehicle 1:1 on link 0

online system 1